[image: image1]



[image: image2.png]



BIDS SPECIFICATIONS FOR STREET PAVING

FOR 

VILLAGE OF LYNCHBURG

2026
1.) Project limits are as follows:
· Park Ave beginning at Herschel Ave through North Ave ending at SR 134  Approximately 2616’x20’
· Patch Work on State Route 135 and Sycamore Street 

· Texas Ave beginning at SR 135 through South Broadway ending at SR 135 Approximately 1015’x22’
       **Note that streets and alleyways will need to be measured and submitted with the bid packet**

2.) All loose material will be cleaned from the existing surface. Also all shoulders should not be higher than the existing pavement prior to paving. The Tack Coat shall be applied in accordance ODOT Spec 407 at a rate of .10Gal/SY.

3.) In no case shall any asphalt course be placed when the air temperature is below 50 degrees F. 

4.) The contractor shall place #404 surface asphalt. The surface overlay depth must be a minimum of two (2) inches of blacktop with one and one half (1 ½) inches compacted on all above streets. Any irregularities or objectionable dips shall be filled during the leveling course. Grinding intersections to butt up with existing intersections to make a seamless transition. 
5.) Asphalt applications will be compacted using equipment and methods in accordance with ODOT Sec. 401.16.

6.) The Contractor will be responsible for supplying, erecting, and maintaining construction warning signs as required by ODOT Spec 614.05.

7.) All bids must be submitted on the Bid Form supplied by the Village of Lynchburg.
8.) All bids must be submitted in a sealed envelope plainly marked “Asphalt Resurfacing.”

9.) Any deviation from these Specifications or the Advertisement for bids may be cause for rejection of the bid.
10.) Satisfactory evidence of insurance must be included. 
11.) Manhole covers and water valves will need to be taken into consideration. Risers, if necessary, will be provided by the contractor. However, bidder will be responsible for making sure the blacktop is level and no sewer manholes and water valves are covered up during the project.
12.)  Bids are due no later than May 14, 2026 by 12:00pm. 
13.) The Village of Lynchburg reserves the right to reject any or all bids and to accept the bid that they determine to be in the best interest of the Village. 

14.) The completion date for the project shall be no later than June 30, 2026
15.) The Contractor shall notify the Village of the project start date at least two (2) weeks in advance of that date. 
16.) The Contractor agrees to save the Village harmless and free from all claims or damages whatsoever nature by reason of work or service performed by Contractor, its agents, servants or employees, who are under Contractor’s control in or upon said Village premises and/or in the furtherance of the said agreement with said Village. 

BID PROPOSAL
To the Village of Lynchburg, Date ___________________________________________ We, ______________________ _________________________________whose address is __________________________________________________________ submit the following bid for furnishing the material listed below as advertised by the Village of Lynchburg. 

Description

  
          
          

  Estimated      
 Unit 
      Total 






 
  Length                  Price
       Price







· Park Ave beginning at Herschel Ave through North Ave ending at SR 134     2616’x20’
       _________________________________________
· Patch Work on State Route 135 and Sycamore Street

                          __________________________________________

· Texas Ave beginning at SR 135 through South Broadway ending at SR 135   1015’x22’
       _________________________________________

**Note that streets may vary with in length and width**




       Street Paving Total        $ __________________________________________________
This material meets the specifications, for same, on file in the Village Office, or has deviations as specifically noted on the attached sheets, which shall be made part of this proposal. 

The completion date for the project shall be no later than June 30, 2026
______________________________________________

Contractor

______________________________________________

Authorized Representative 

_________________

Date

______________________________________________

Witness

Date

______________________________________________

Witness

Date
VILLAGE OF LYNCHBURG


155 S. Main St. ~ P.O. Box 402  


Lynchburg, OH 45142                        


Phone:  937-364-2241 - Fax: 937-364-6431


� HYPERLINK "http://www.lynchburgohio.org" �www.lynchburgohio.org�


  











